Complete Line-by-Line Explanation

1. accelerator_top.sv - Main Accelerator Module

verilog

`timescale 1ns / 1ps

- Sets simulation time unit to 1 nanosecond with 1 picosecond precision

verilog

module accelerator_top #(

parameter integer WORD_SIZE  = 64,

parameter integer PE_ROWS    = 8,

parameter integer PE_COLS    = 8,

parameter integer SRAM_DEPTH = 64

)(

- Defines the top-level module with configurable parameters:

  - WORD_SIZE=64: Each weight/activation is 64 bits (64 binary values)

  - PE_ROWS=8, PE_COLS=8: Creates an 8×8 grid = 64 Processing Elements

  - SRAM_DEPTH=64: Each SRAM holds 64 words

verilog

input  wire                   clk,

input  wire                   reset,

input  wire                   start,

output reg                    done,

- Control signals:

  - clk: Clock signal for synchronous logic

  - reset: Active-high reset (clears all registers)

  - start: Pulse high for 1 cycle to begin computation

  - done: Goes high when computation finishes

verilog

input  wire                   wr_en,

input  wire [5:0]             wr_addr,

input  wire [WORD_SIZE-1:0]   wr_data,

input  wire                   wr_type

);

- Write interface for loading data (used by testbench):

  - wr_en: Write enable (1=write, 0=no write)

  - wr_addr: 6-bit address (0-63) for which location to write

  - wr_data: 64-bit data to write

  - wr_type: 0=write to weight SRAM, 1=write to activation SRAM

verilog

localparam integer NUM_PES = PE_ROWS*PE_COLS;

localparam integer PCW     = $clog2(WORD_SIZE+1);

- Local parameters (constants):

  - NUM_PES = 8×8 = 64: Total number of PEs

  - PCW = $clog2(65) = 7: Popcount Width - bits needed to represent 0-64 (7 bits can hold 0-127)

verilog

// SRAM arrays

reg [WORD_SIZE-1:0] weight_sram    [0:SRAM_DEPTH-1];

reg [WORD_SIZE-1:0] activation_sram[0:SRAM_DEPTH-1];

- Two SRAM memory arrays:

  - weight_sram: 64 locations, each 64 bits wide - stores weight values

  - activation_sram: 64 locations, each 64 bits wide - stores activation values

  - Implemented as register arrays in hardware

verilog

// Writes

always @(posedge clk) begin

if (wr_en) begin

if (!wr_type) weight_sram[wr_addr] <= wr_data;

else activation_sram[wr_addr] <= wr_data;

end

end

- Write logic (synchronous):

  - On every clock rising edge, if write is enabled:

    - If wr_type=0: write wr_data to weight SRAM at address wr_addr

    - If wr_type=1: write wr_data to activation SRAM at address wr_addr

  - Uses non-blocking assignment (<=) for synchronous logic

verilog

// FSM

localparam [2:0] S_IDLE=3'd0, S_LOAD=3'd1, S_RUN=3'd2, S_DONE=3'd3;

reg [2:0] state, nstate;

- Finite State Machine (FSM) with 4 states:

  - S_IDLE (0): Waiting for start signal

  - S_LOAD (1): Prepare for computation (1 cycle)

  - S_RUN (2): Processing data through PEs

  - S_DONE (3): Computation complete

- state: Current state register

- nstate: Next state (combinational logic)

verilog

reg [7:0] rd_idx;

reg [7:0] run_cycles;

reg       ce_array;

- Control registers:

  - rd_idx: 8-bit read index (0-63) - which SRAM word to read

  - run_cycles: Counts how many cycles we've been in RUN state

  - ce_array: Clock Enable for PE array (1=active, 0=idle)

verilog

wire [NUM_PES*WORD_SIZE-1:0] w_flat, a_flat, m_flat;

wire [NUM_PES*PCW-1:0]       pc_flat;

reg  [31:0]                  total_popcount;

- Data signals:

  - w_flat: Flattened weights for all PEs (64 PEs × 64 bits = 4096 bits)

  - a_flat: Flattened activations for all PEs (4096 bits)

  - m_flat: Valid bit masks for all PEs (4096 bits)

  - pc_flat: Flattened popcount outputs from all PEs (64 PEs × 7 bits = 448 bits)

  - total_popcount: 32-bit accumulator for final result

verilog

// Broadcast same word to all PEs

genvar k;

generate

for (k=0;k<NUM_PES;k=k+1) begin: feed

assign w_flat[(k+1)*WORD_SIZE-1 : k*WORD_SIZE] = weight_sram[rd_idx];

assign a_flat[(k+1)*WORD_SIZE-1 : k*WORD_SIZE] = activation_sram[rd_idx];

assign m_flat[(k+1)*WORD_SIZE-1 : k*WORD_SIZE] = {WORD_SIZE{1'b1}};

end

endgenerate

- Generate block - creates hardware at synthesis time:

  - Loop creates 64 connections (k=0 to 63)

  - Each PE gets the same word from SRAM (broadcast architecture):

    - PE 0 gets bits [63:0] of w_flat, assigned from weight_sram[rd_idx]

    - PE 1 gets bits [127:64] of w_flat, also from weight_sram[rd_idx]

    - All 64 PEs process the same weight/activation pair

  - m_flat sets all mask bits to 1 (all bits valid)

  - {WORD_SIZE{1'b1}} replicates 1 64 times = 64'hFFFFFFFFFFFFFFFF

verilog

pe_array #(

.ROWS(PE_ROWS), .COLS(PE_COLS), .WORD_SIZE(WORD_SIZE), .PIPELINE(1)

) peA (

.clk(clk), .ce(ce_array),

.weights_in_flat(w_flat),

.activations_in_flat(a_flat),

.valid_mask_flat(m_flat),

.popcounts_out_flat(pc_flat)

);

- PE Array instantiation:

  - Creates the 8×8 PE array module

  - Parameters: 8 rows, 8 columns, 64-bit words, 1-cycle pipeline

  - Inputs: clock, enable, flattened weights/activations/masks

  - Output: flattened popcounts (one 7-bit value per PE)

  - Takes 1 clock cycle to produce output (pipeline delay)

verilog

// Extract popcounts

wire [PCW-1:0] pc_array [0:NUM_PES-1];

generate

for (k=0; k<NUM_PES; k=k+1) begin: extract_pc

assign pc_array[k] = pc_flat[(k+1)*PCW-1 : k*PCW];

end

endgenerate

- Unpack flattened popcounts into an array:

  - pc_array[0] gets bits [6:0] (PE 0's popcount)

  - pc_array[1] gets bits [13:7] (PE 1's popcount)

  - Creates 64 separate 7-bit wires for easier access

verilog

// Adder tree

wire [PCW+1:0] sum_stage1 [0:31];

wire [PCW+2:0] sum_stage2 [0:15];

wire [PCW+3:0] sum_stage3 [0:7];

wire [PCW+4:0] sum_stage4 [0:3];

wire [PCW+5:0] sum_stage5 [0:1];

wire [12:0] cycle_sum;

- Multi-stage adder tree wires:

  - Stage 1: 32 sums (8 bits each) - pairs of PEs added

  - Stage 2: 16 sums (9 bits each)

  - Stage 3: 8 sums (10 bits each)

  - Stage 4: 4 sums (11 bits each)

  - Stage 5: 2 sums (12 bits each)

  - cycle_sum: Final 13-bit sum of all 64 PE outputs

verilog

generate

for (k=0; k<32; k=k+1) begin: stage1

assign sum_stage1[k] = pc_array[2*k] + pc_array[2*k+1];

end

- Stage 1: Add pairs of adjacent popcounts:

  - sum_stage1[0] = pc_array[0] + pc_array[1]

  - sum_stage1[1] = pc_array[2] + pc_array[3]

  - ... 32 additions total

  - Result: 64 values → 32 values

verilog

for (k=0; k<16; k=k+1) begin: stage2

assign sum_stage2[k] = sum_stage1[2*k] + sum_stage1[2*k+1];

end

- Stage 2: Add pairs from stage 1:

  - 32 values → 16 values

verilog

for (k=0; k<8; k=k+1) begin: stage3

assign sum_stage3[k] = sum_stage2[2*k] + sum_stage2[2*k+1];

end

for (k=0; k<4; k=k+1) begin: stage4

assign sum_stage4[k] = sum_stage3[2*k] + sum_stage3[2*k+1];

end

for (k=0; k<2; k=k+1) begin: stage5

assign sum_stage5[k] = sum_stage4[2*k] + sum_stage4[2*k+1];

end

endgenerate

assign cycle_sum = sum_stage5[0] + sum_stage5[1];

- Stages 3-5: Continue binary tree reduction:

  - Stage 3: 16 → 8

  - Stage 4: 8 → 4

  - Stage 5: 4 → 2

  - Final: 2 → 1 (cycle_sum)

- This creates a balanced adder tree for fast, synthesizable summation

- Maximum value: 64 PEs × 64 bits = 4096 (needs 13 bits)

verilog

// FSM sequential

always @(posedge clk or posedge reset) begin

if (reset) state <= S_IDLE;

else state <= nstate;

end

- FSM state register:

  - Asynchronous reset: If reset goes high, immediately go to IDLE

  - Otherwise: On clock edge, update current state to next state

  - Standard Moore machine state register

verilog

// Datapath

always @(posedge clk or posedge reset) begin

if (reset) begin

rd_idx <= 8'd0;

run_cycles <= 8'd0;

ce_array <= 1'b0;

total_popcount <= 32'd0;

done <= 1'b0;

- Datapath logic (sequential):

  - Reset: Initialize all registers to zero/idle

  - 8'd0 means 8-bit decimal 0

  - 32'd0 means 32-bit decimal 0

verilog

end else begin

done <= 1'b0;

case (state)

S_IDLE: begin

total_popcount <= 32'd0;

rd_idx <= 8'd0;

run_cycles <= 8'd0;

ce_array <= 1'b0;

end

- S_IDLE state:

  - Clear accumulator

  - Reset read index to 0

  - Reset cycle counter

  - Disable PE array

  - Wait for start signal

verilog

S_LOAD: begin

ce_array <= 1'b1;

rd_idx <= 8'd0;

run_cycles <= 8'd0;

end

- S_LOAD state (1 cycle):

  - Enable PE array (clock enable)

  - Set read index to 0

  - Reset run cycle counter

  - Purpose: Start pipeline, feed first data to PEs

verilog

S_RUN: begin

run_cycles <= run_cycles + 8'd1;

// Accumulate after first cycle (pipeline delay)

if (run_cycles > 8'd0 && run_cycles <= SRAM_DEPTH) begin

total_popcount <= total_popcount + cycle_sum;

end

- S_RUN state - Accumulation:

  - Increment cycle counter each clock

  - Skip first cycle (run_cycles > 0): Pipeline hasn't filled yet

  - Cycles 1-64: Accumulate cycle_sum into total_popcount

  - This handles the 1-cycle PE pipeline delay correctly

verilog

// Increment address

if (rd_idx < SRAM_DEPTH - 1) begin

rd_idx <= rd_idx + 8'd1;

end

end

- S_RUN state - Address increment:

  - If not at last address (63), increment to next

  - Reads: 0, 1, 2, ..., 63

  - Stays at 63 after reaching it

verilog

S_DONE: begin

done <= 1'b1;

ce_array <= 1'b0;

end

endcase

end

end

- S_DONE state:

  - Set done flag high (tells testbench we're finished)

  - Disable PE array to save power

  - Result is in total_popcount

verilog

// FSM combinational

always @(*) begin

nstate = state;

case (state)

S_IDLE: if (start) nstate = S_LOAD;

S_LOAD: nstate = S_RUN;

S_RUN:  if (run_cycles == SRAM_DEPTH + 1) nstate = S_DONE;

S_DONE: if (!start) nstate = S_IDLE;

endcase

end

endmodule

- FSM next-state logic (combinational):

  - Default: stay in current state

  - IDLE → LOAD: When start pulse received

  - LOAD → RUN: Always (after 1 cycle)

  - RUN → DONE: When run_cycles == 65 (after 64 accumulations)

  - DONE → IDLE: When start goes back low (ready for next computation)

2. pe_array.sv - Processing Element Array

verilog

`timescale 1ns / 1ps

module pe_array #(

parameter integer ROWS      = 8,

parameter integer COLS      = 8,

parameter integer WORD_SIZE = 64,

parameter integer PIPELINE  = 1

)(

- Wrapper module for array of PEs

- Parameters same as accelerator_top

- PIPELINE=1: Each PE has 1-cycle pipeline

verilog

input  wire                         clk,

input  wire                         ce,

input  wire [ROWS*COLS*WORD_SIZE-1:0] weights_in_flat,

input  wire [ROWS*COLS*WORD_SIZE-1:0] activations_in_flat,

input  wire [ROWS*COLS*WORD_SIZE-1:0] valid_mask_flat,

output wire [ROWS*COLS*$clog2(WORD_SIZE+1)-1:0] popcounts_out_flat

);

- Inputs/outputs for all PEs flattened into single vectors:

  - ce: Clock enable for all PEs

  - weights_in_flat: 4096 bits (64 words × 64 bits)

  - activations_in_flat: 4096 bits

  - valid_mask_flat: 4096 bits

  - popcounts_out_flat: 448 bits (64 × 7 bits)

verilog

localparam integer NUM_PES = ROWS*COLS;

localparam integer PCW     = $clog2(WORD_SIZE+1);

genvar i;

generate

for (i = 0; i < NUM_PES; i = i + 1) begin : pe_gen

- Generate 64 PE instances (i = 0 to 63)

- Each loop iteration creates one PE

verilog

xpe_core #(

.WORD_SIZE (WORD_SIZE),

.PIPELINE  (PIPELINE),

.HAS_MASK  (1)

) xpe_i (

- Instantiate one xpe_core (PE) with:

  - 64-bit words

  - 1-cycle pipeline

  - Mask support enabled

verilog

.weight_word_in     (weights_in_flat     [ (i+1)*WORD_SIZE-1 : i*WORD_SIZE ]),

.activation_word_in (activations_in_flat [ (i+1)*WORD_SIZE-1 : i*WORD_SIZE ]),

.valid_mask_in      (valid_mask_flat     [ (i+1)*WORD_SIZE-1 : i*WORD_SIZE ]),

- Connect PE inputs by slicing the flattened vectors:

  - PE 0: bits [63:0]

  - PE 1: bits [127:64]

  - PE i: bits [(i+1)×64-1 : i×64]

verilog

.clk                (clk),

.ce                 (ce),

.popcount_out       (popcounts_out_flat  [ (i+1)*PCW-1 : i*PCW ])

);

end

endgenerate

endmodule

- Connect clock, enable, and output

- Output sliced similarly: PE i writes to bits [(i+1)×7-1 : i×7]

3. xpe_core.sv - Single Processing Element

verilog

`timescale 1ns / 1ps

module xpe_core #(

parameter integer WORD_SIZE       = 64,

parameter integer PIPELINE        = 1,

parameter integer HAS_MASK        = 1

)(

input  wire [WORD_SIZE-1:0] weight_word_in,

input  wire [WORD_SIZE-1:0] activation_word_in,

input  wire [WORD_SIZE-1:0] valid_mask_in,

input  wire                  clk,

input  wire                  ce,

output reg  [$clog2(WORD_SIZE+1)-1:0] popcount_out

);

- Single PE that performs XNOR-popcount operation

- Inputs: 64-bit weight, 64-bit activation, 64-bit mask

- Output: 7-bit popcount (0-64)

verilog

localparam integer PCW = $clog2(WORD_SIZE+1);

wire [WORD_SIZE-1:0] xnor_result_raw = ~(weight_word_in ^ activation_word_in);

- XNOR operation:

  - weight_word_in ^ activation_word_in: XOR (bits differ = 1, same = 0)

  - ~(...): NOT (invert) → XNOR (bits same = 1, differ = 0)

  - Result: 64-bit vector where 1 = matching bits

verilog

wire [WORD_SIZE-1:0] xnor_result     = (HAS_MASK) ? (xnor_result_raw & valid_mask_in) : xnor_result_raw;

- Apply mask if enabled:

  - If HAS_MASK=1: AND with mask (only count masked bits)

  - If HAS_MASK=0: Use raw XNOR result

  - This allows ignoring certain bit positions

verilog

// Balanced adder tree popcount

function [PCW-1:0] popcount64;

input [WORD_SIZE-1:0] v;

integer j;

reg [PCW-1:0] s;

begin

s = {PCW{1'b0}};

for (j = 0; j < WORD_SIZE; j = j + 1) begin

s = s + v[j];

end

popcount64 = s;

end

endfunction

- Popcount function:

  - Takes 64-bit input v

  - Initialize sum s to 0

  - Loop through each bit: add bit value (0 or 1) to sum

  - Returns count of 1's in the input

  - Synthesizable: Tools convert this to optimized adder tree

verilog

wire [PCW-1:0] pc_comb = popcount64(xnor_result);

- Calculate popcount combinationally

- pc_comb holds number of matching bits

verilog

generate

if (PIPELINE != 0) begin : gen_pipe

always @(posedge clk) begin

if (ce) popcount_out <= pc_comb;

end

- Pipelined mode (PIPELINE=1):

  - Register the popcount output

  - Only updates when clock enable is high

  - Adds 1-cycle latency

verilog

end else begin : gen_comb

always @(*) begin

popcount_out = pc_comb;

end

end

endgenerate

endmodule

- Combinational mode (PIPELINE=0):

  - Output directly = calculated popcount

  - No register, no latency

  - We use PIPELINE=1 for better timing

4. tb_accelerator_top.sv - Main Testbench

verilog

`timescale 1ns / 1ps

module tb_accelerator_top;

localparam WORD_SIZE=64, SRAM_DEPTH=64, PE_ROWS=8, PE_COLS=8;

- Testbench module (no ports)

- Same parameters as design under test (DUT)

verilog

reg clk, reset, start;

wire done;

reg wr_en;

reg [5:0] wr_addr;

reg [WORD_SIZE-1:0] wr_data;

reg wr_type;

- Test signals:

  - reg for inputs (testbench drives these)

  - wire for outputs (DUT drives these)

verilog

accelerator_top #(

.WORD_SIZE(WORD_SIZE),

.PE_ROWS(PE_ROWS),

.PE_COLS(PE_COLS),

.SRAM_DEPTH(SRAM_DEPTH)

) dut (

.clk(clk),

.reset(reset),

.start(start),

.done(done),

.wr_en(wr_en),

.wr_addr(wr_addr),

.wr_data(wr_data),

.wr_type(wr_type)

);

- Instantiate DUT (Device Under Test):

  - Connect testbench signals to accelerator ports

  - Named port connections for clarity

verilog

always #5 clk = ~clk;

- Clock generator:

  - Toggle clock every 5ns

  - Period = 10ns → Frequency = 100 MHz

  - #5 is a delay statement

verilog

integer i, NUM_PES, N, signed_result, pc_total;

integer j, manual_count;

reg [63:0] w0, a0, xnor_result;

- Test variables (declared at top):

  - Loop counters, calculation variables

  - Must be declared before initial begin

verilog

initial begin

$display("=== Starting BNN Accelerator Testbench ===");

clk = 0;

reset = 1;

start = 0;

wr_en = 0;

wr_addr = 0;

wr_data = '0;

wr_type = 0;

- Initialize all signals:

  - Clock starts at 0

  - Assert reset

  - All other signals idle

  - '0 means "all bits zero" (width inferred)

verilog

repeat(2) @(posedge clk);

reset = 0;

$display("Time=%0dns: Reset deasserted", $time/1000);

- Release reset:

  - Wait for 2 clock edges (20ns)

  - Deassert reset

  - $time gives time in picoseconds, divide by 1000 for nanoseconds

  - $display prints to console

verilog

// Load weights

@(posedge clk);

wr_en = 1;

wr_type = 0;

$display("Time=%0dns: Loading weights...", $time/1000);

for (i = 0; i < SRAM_DEPTH; i = i + 1) begin

wr_addr = i;

wr_data = (i == 0) ? 64'h589A86C459345B3C : 64'h0;

@(posedge clk);

end

- Load weight SRAM:

  - Enable write, select weight SRAM (type=0)

  - Loop through all 64 addresses

  - Address 0: Load test pattern 0x589A86C459345B3C

  - Addresses 1-63: Load zeros

  - Wait for clock edge after each write (synchronous SRAM)

verilog

wr_en = 0;

@(posedge clk);

$display("Time=%0dns: Weights loaded", $time/1000);

$display("  weight[0] = %h", dut.weight_sram[0]);

$display("  weight[1] = %h", dut.weight_sram[1]);

- Finish writing:

  - Disable write

  - Wait one more clock (ensure last write completes)

  - Display values using hierarchical reference: dut.weight_sram[0]

  - %h prints hexadecimal

verilog

// Load activations

@(posedge clk);

wr_en = 1;

wr_type = 1;

$display("Time=%0dns: Loading activations...", $time/1000);

for (i = 0; i < SRAM_DEPTH; i = i + 1) begin

wr_addr = i;

wr_data = (i == 0) ? 64'h6FA49326961A604D : 64'h0;

@(posedge clk);

end

wr_en = 0;

@(posedge clk);

- Load activation SRAM:

  - Same process as weights

  - Select activation SRAM (type=1)

  - Address 0: Load 0x6FA49326961A604D

  - Addresses 1-63: Load zeros

verilog

// Run computation

@(posedge clk);

start = 1;

$display("\nTime=%0dns: Start signal asserted", $time/1000);

@(posedge clk);

start = 0;

- Start computation:

  - Wait for clock edge

  - Pulse start high for 1 cycle

  - Wait another clock edge

  - Deassert start

verilog

// Wait and check progress

repeat(5) @(posedge clk);

$display("\nTime=%0dns: During computation:", $time/1000);

$display("  state = %0d (2=RUN)", dut.state);

$display("  rd_idx = %0d", dut.rd_idx);

$display("  w_flat[63:0] (to PE0) = %h", dut.w_flat[63:0]);

$display("  a_flat[63:0] (to PE0) = %h", dut.a_flat[63:0]);

$display("  pc_array[0] (PE0 output) = %0d", dut.pc_array[0]);

$display("  cycle_sum = %0d", dut.cycle_sum);

$display("  total_popcount = %0d", dut.total_popcount);

- Monitor progress:

  - Wait 5 clock cycles

  - Sample internal signals (hierarchical access)

  - Display state machine state, indices, data values

  - Useful for debugging

verilog

wait(done == 1);

@(posedge clk);

$display("\nTime=%0dns: Done signal received!", $time/1000);

- Wait for completion:

  - wait() blocks until condition true

  - Wait for done signal from DUT

  - Synchronize to clock edge

verilog

NUM_PES = PE_ROWS * PE_COLS;

N = NUM_PES * SRAM_DEPTH * WORD_SIZE;

pc_total = dut.total_popcount;

signed_result = (2 * pc_total) - N;

- Calculate results:

  - Total PEs = 64

  - Total bits = 64 PEs × 64 words × 64 bits = 262,144

  - Read popcount from DUT

  - Signed result = 2×popcount - N (BNN formula)

verilog

$display("\n=== RESULTS ===");

$display("Total popcount = %0d", pc_total);

$display("N (total bits) = %0d", N);

$display("Signed result (2*pc-N) = %0d", signed_result);

- Display results:

  - Show all calculated values

  - This is what we check for correctness

verilog

// Manual calculation for first word

w0 = 64'h589A86C459345B3C;

a0 = 64'h6FA49326961A604D;

xnor_result = ~(w0 ^ a0);

manual_count = 0;

for (j = 0; j < 64; j = j + 1) begin

if (xnor_result[j]) manual_count = manual_count + 1;

end

- Manual verification:

  - Recalculate XNOR of first word pair

  - Count 1's manually using loop

  - Compare with DUT result

verilog

$display("\n=== Manual Verification (First Word) ===");

$display("w[0] = %h", w0);

$display("a[0] = %h", a0);

$display("XNOR = %h", xnor_result);

$display("Expected popcount for one PE on first word = %0d", manual_count);

$display("Expected for all 64 PEs = %0d", manual_count * 64);

$display("Actual result includes 63 zero-XNOR-zero = 64*64 = 4096 per word");

$display("Total expected = %0d + (63 * 4096) = %0d", (manual_count * 64), ((manual_count * 64) + (63 * 4096)));

$display("===============\n");

- Show expected calculation:

  - First word: 28 matching bits × 64 PEs = 1,792

  - Remaining 63 words: all zeros, so 0 XNOR 0 = all 1's = 64 × 64 × 63 = 258,048

  - Total expected: 259,840

verilog

#100;

$finish;

end

endmodule

- End simulation:

  - Wait 100ns

  - $finish ends simulation

  - Returns control to simulator

Key Concepts Summary

1. BNN Operation (XNOR-Popcount)

- Binary weights/activations: Each bit is -1 or +1

- Multiplication: -1×-1=1, 1×1=1, -1×1=-1, 1×-1=-1

- In binary: Same bits → +1, Different bits → -1

- XNOR: Same bits = 1, Different = 0

- Popcount: Count of 1's = count of matching bits

- Signed result: 2×popcount - N (converts 0-N range to -N to +N)

2. Pipeline

- PEs have 1-cycle delay (registered output)

- Data enters on cycle N, result available on cycle N+1

- Must skip first accumulation (invalid data)

- Must add extra cycle at end (capture last result)

3. Broadcast Architecture

- All 64 PEs process the same weight/activation pair

- Advantage: Simple interconnect, saves routing

- Result: 64× parallelism on same data

4. Generate Blocks

- Creates hardware at synthesis time (not runtime)

- Used for: PE array, adder tree, unpacking vectors

- More efficient than procedural loops

5. FSM Design

- IDLE: Wait for start

- LOAD: Fill pipeline (1 cycle)

- RUN: Process all 64 words, accumulate results

- DONE: Signal completion

SINCE OUR CURRENT ARCHITECTURE CAN ONLY DO THE REDUNDANT COMPUTATION WE WILL MOVE ONTO A MAJOR ARCHOTECTURE UPGRADE

All PEs get same data → 64× redundant computation

PE[0]: w[0] • a[0]

PE[1]: w[0] • a[0]  ← Same!

...

PE[63]: w[0] • a[0] ← Same!

THE SAME DATA IS LOADED INTO ALL THE 64 PE Arrays AT THE SAME TIME WHICH IS OF NO USE AS WE WOULD NEED TO AN EFFICIENT COMPUTATION DEVICE SO WE WOULD REQUIRE SYSTOLIC COMPUTATION . SIMILAR TO THE ONE MENTIONED BELOW :-

Each PE computes one output

PE[0]:  w[0,0]•a[0] + w[0,1]•a[1] + ... + w[0,63]•a[63] → out[0]

PE[1]:  w[1,0]•a[0] + w[1,1]•a[1] + ... + w[1,63]•a[63] → out[1]

...

PE[63]: w[63,0]•a[0] + w[63,1]•a[1] + ... + w[63,63]•a[63] → out[63]

close_sim

file delete -force E:/project_2/project_2.sim/sim_1/behav/xsim/xsim.dir

launch_simulation

run 50us